PRGRAMMING THE NOAA WR-100 MIDLAND SAME WEATHER RADIO

Programming the Radio for Your Area (Channel 1 thru 7)

Install batteries (if batteries are not available…plug radio in)

- Turn on Radio set volume so that static isn’t too loud

- Press MENU

- Press the UP arrow button until you find CHANNEL…..then press SELECT

- Press “UP” to choose channel 1-7. Choose the channel with best quality of sound.

- Press SELECT

- Press MENU….THIS ENDS THE CHANNEL ENTRY PROCESS!!!

To Set FIPS Code

- Turn on Radio set volume so that static isn’t too loud

- Press MENU

- Press UP arrows to scroll to option that reads SAME SET is displayed.  Then Press SELECT

- Press UP arrows to scroll through SINGLE, MULTIPLE or ANY

- Display SINGLE press SELECT, SAME 01 will be displayed

- Press SELECT using UP arrow find first digit of SAME county code.  Press RIGHT to save digit (FIPS code for Montgomery Co. = 021173

- Do the rest of the digits the same way

- Press SELECT to store count code…. press MENU to close

To set Multiple FIPS Codes

- Turn on Radio set volume so that static isn’t too loud

- Press MENU

- Press UP arrows to scroll to option that reads SAME SET is displayed.  Then Press SELECT

- Press UP arrows to scroll through SINGLE, MULTIPLE or ANY

- Display MULTIPLE then press SELECT…  SAME 01 will display.

- Since Montgomery county FIPS code has already been entered….press UP arrow to advance to SAME 02 Program ….press SELECT

- Entry Bourbon county FIPS code 021017….then press SELECT

- Press UP arrow to SAME 03…press SELECT

- Enter Clark county’s FIPS code 021049…..then press SELECT

- Press MENU

- Press MENU

- All county FIPS codes are at: http://www.crh.noaa.gov/pah/nwr/same-ky.php